Dave Ng’ang’a
Engineering Notebook

Computer Organization and Architecture

ASTEROID FIELD

Objective: Creating a more primitive version of chicken invaders, the idea being the user pilots a spaceship that must navigate an asteroid field. The ship is fixed with weapons and infinite ammunition and can therefore shoot and destroy asteroid fields.

The game will cycle through several songs that will increase in pitch and tempo as the game proceeds (Indicating an increase in difficulty)

Pieces for use:

- Arduino board (mega 2560)

- 3 led lights. (For hp tracking)

- 8x8 Matrix (The stage for the game)

- LCD (displaying information about the game – High Score, etc)

- Passive buzzer (play the music that will accompany the game)

- Joystick Module (To pilot the spaceship and fire)

@ 11/23/2021 (1 Hour)

Objective: Decided to begin learning how the joystick module works.

  - https://youtu.be/MlDi0vO9Evg - This provides a lot of details on how to the joystick works

  - It appears the Joystick functions in both x and y axis so I’d need to make sure my code listens only for the y axis (the spaceship cannot move forward)

    - There are two separate pins for the x and y axis. It’d probably be easiest to just not even plug the x axis pin.

  - I’m yet to learn how to map a function (fire weapons) to the button click on the joystick.

  - https://www.best-microcontroller-projects.com/arduino-joystick.html

    - This link guides me to a GitHub account with a library on the Arduino joystick,

      - The Github has some very useful commands I can use abstractly

        - isPress() – Check if button has been pressed

        - isUp(), isDown(), isRight(), isLeft() – return a Boolean variable that corresponds to the direction the joystick has been pushed in.

        - spent several minutes reading through this library. I have it downloaded and will probably incorporate it into my project.

      - In one of the example files, I found a switch statement that incorporates the different joystick functions (figure 1)

Figure 1

Objective: Find information on ways to use the 8x8 matrix optimally

Since the 8x8 matrix is essentially my game screen it needs to be the one I have the most solid and smoothest understanding in:

- As I remember from previous projects. The 8x8 matrix could be used by the manipulation of binary digits. (a set of 8 binary digits with 8 places corresponding with the rows and columns of the matrix).

- The following website provides detail on how to program the matrix: https://create.arduino.cc/projecthub/SAnwandter1/programming-8x8-led-matrix-23475a

@ 11/30/2021 (1 Hour)

Objective: I already have a decent understanding of all the other aspects of the objects in this project except the led. I’m going to attempt to set it up.

- Failed to get LCD working, all I get is the single letter f.

@ 12/2/2021 (1 Hour)

- The moodiness of computers never ceases to amaze me. Something was wrong with the code I had. Something the compiler wasn’t picking. I was able to get the LCD working by completely changing the file with my code.

- LCD is functioning now.

Objective: I need to get the buzzer working and start playing tunes out of it

- Update - Buzzer is working.

- NEW IDEA

  - Use a speaker instead of a buzzer. Improved sound quality and you might be able to create harmonies. Though I’m not sure how as it will only be able to play one note at a time.

    - Link to coffin dance Arduino uno code: - https://github.com/Rahul24-06/Arduino-Coffin-Dance Theme/blob/master/Coffin_dance_arduino/Coffin_dance_arduino.ino

@ 12/7/2021 (1 Hour)

- Objective: Attempting to incorporate a speaker instead of a buzzer. I found the speaker from an old pair of headphones.

  - SUCCESS!

- Objective: Finally Begin the main logic on the 8x8 matrix

    - Note: Classmates have been kind enough to lend me their 8x8’s so I can make it a 4x4 daisy chain of matrixes.

  - Creating a spaceship:

  - Creating an asteroid:

  - Looping all into a game cycle:

@12/8/2021(1 Hour)

- Continuing the above objectives:

  - It just dawned on me that I can plug the 8x8 matrices directly onto the fretboard eliminating half the cablings I would need.

@12/9/2021(4 hours)

- I’m thinking two 8x8 matrices are going to have to do. I’m currently working on daisy chaining the two led matrices as well as creating the initial game loop

- https://create.arduino.cc/projecthub/goldscrew/make-a-led-matrix-box-with-multiple-effects-da3203#code

  - The link above shows daisy chaining in up to 4 matrices. I installed the required libraries and gave it a try

    - Failed

- https://www.brainy-bits.com/post/scrolling-text-using-the-max7219-led-dot-matrix

  - This my second source at attempting to get the daisy chains working.

    - Had to include a separate zip downloaded library file

      - Success!

    - I had to switch the order of the matrixes though, in order to get movement moving the right way.

  - Spaceship

    - Ok so I need to be able to spawn a spaceship and keep it constant in light of moving asteroids.

    - Upon further contemplating, Daisy chaining is not it. Instead, I will need to connect each led matrix separately. Because there’s no other way to emulate the game across two matrices since daisy chaining involves bit shifting and what I need is control over all 16 rows and columns

  - Asteroids

@12/10/2021(8 hours)

- I had to rewire everything. The importance of neat cabling cannot be overstated.

- I also got a cardboard floor and glued, and zip tied the project onto it

- It’s been a long day

  - I had issues with the 8x8 matrix that boiled down to improper wiring.

  - I chose to get rid of the lcd aspect of the project.

    - I don’t really need it

    - I don’t have the space or cables to include it together with everything else.

- I soldered two cables onto the speaker and tested it to make sure it works. I have the skeleton of the game forming. The spaceship I have been able to move around using bit shift operations. Now I need to tie those bit shift operations to a joystick movement.

  - Success!

- Now I’m trying to incorporate the fire function. Where the joystick is pressed, and the ship fires a missile.

  - This is literally impossible.

  - Arduino has no multithreading capabilities so other than being able to use the millis() function in ways I don’t have the time to figure out, I don’t see how I’d be able to incorporate a separate function that traces each bullet fired by the ship

- Closing update for today

  - I have been able to create asynchronous functions where the spaceship can move as well as the asteroids.

    - I did this through the use of the method millis() over delay().

  - My next steps will be getting the asteroid to travel from one matrix to the next and then eventually working out the interaction of when the spaceship and an asteroid collide.

@12/11/2021(4 hours)

- I was able to get the asteroids to travel across matrixes. Now I’m attempting to provide an interaction when the asteroid runs into the spaceship.

- https://stackoverflow.com/questions/10493411/what-is-bit-masking

  - Bit masking is what I’d use, to check if the asteroid will collide with the spaceship on next tick.

  - It worked!

@12/11/2021(4 hours)

- I incorporated the led lights onto the project and cleaned finished up the masking concept to check for asteroid and spaceship collisions.

Summary

I have grossly overestimated the amount of work I would be able to do for this project, But I have a working version of the game:

    - The controller moves the spaceship on the matrix however the spaceship is not able to fire any weapons.

    - Sound effects are played but there is no song accompanying the video game as initially intended.

    - Led lights track the health bar of the spaceship and when the spaceship collides with an asteroid three times. It is ruined and the game ends.

    - Below is a visual of the wiring of the project, and below it a break down of the wiring.

    - (LedControl.h was the only library included in the project)

- Joystick – (Plug to ground and 5V)

  - MS  Pin 3

  - VRx  pin A3 (VRy is not needed)

- Speaker Module (plug negative to ground)

  - Positive  pin 2

- 2x(8x8 LED Matrix)

  - Matrix 1 (VCC to positive and ground to ground)

    - Din  pin 8

    - Cs  pin 9

    - Clk  pin 10

  - Matrix 2 (VCC to positive and ground to ground)

    - Din  pin 12

    - Cs  pin 11

    - Clk pin 13

- 3x Led Lights (negative to negative)

  - LED1 - Positive end (long end)  pin 4 and 220ohm resistor

  - LED2 - Positive end (long end)  pin 5 and 220ohm resistor

  - LED3 - Positive end (long end)  pin 7 and 220ohm resistor